# Task Title: Interpret Electrical Measuring Equipment Readings

# OALCF Cover Sheet – Practitioner Copy

Learner Name: __________________________________________

Date Started: ___________________________________________

Date Completed: _________________________________________

| Goal Path: | Employment | Apprenticeship |
| --- | --- | --- |
| Secondary School | Post Secondary | Independence |

Successful Completion:  Yes  	     No

Task Description: The learner will find and interpret information about electrical measuring equipment and processes.

Main Competency/Task Group/Level Indicator:

- Find and Use Information/Read continuous text/A1.3

- Find and Use Information/Interpret documents/A2.2

- Find and Use Information/Extract info from films, broadcasts and presentations/A3

- Understand and Use Numbers/Use measures/C3.1

- Use Digital Technology/D.1

Materials Required:

- Pen/pencil and paper

- Computer or digital device

# Learner Information

Electricians use electrical measuring equipment to test systems and components.  They use the readings to determine if a component needs to be replaced, if a system has been safely shut down (lock out and tag out), and to determine the voltage, amperage, and wattage of systems and components.

Scan the “Multimeter Images”.

# Work Sheet

Task 1a:  What colour is used to measure the ohms of a circuit?

Answer:

_____________________________________________________________

Task 1b:  Circle on the picture where the meter setting dial should be set to measure a 9 Volt battery.

Answer: No written response required here.

Task completed:  Yes:

_____________________________________________________________

Task 1c:  Electricians always set the meter slightly higher than the expected voltage being measured.  Circle on the picture where the dial should be set to measure a 240 volt AC circuit.

Answer: No written response required here.

Task completed:  Yes:

_____________________________________________________________

Task 1d:  Batteries are considered in working order at 70% voltage or better.  A 9 Volt battery reading shows the needle at about 43%.  In what range will the needle point?

Answer:

______________________________________________________

Go to the Lockout/Tag out page on the Canadian Centre for Occupational Health and Safety website:   http://www.ccohs.ca/oshanswers/hsprograms/lockout.html

Task 2a:  List three key pieces of information to be included on a tag?

Answer:

_____________________________________________________________

Task 2b:  An electrician uses a voltmeter to ensure the power to a circuit is safe to complete the lock out/tag out procedure.  Explain what the voltage reading should be and why this is the case.

Answer:

______________________________________________________

Watch this video about using digital and analog ohmmeters:  http://www.youtube.com/watch?v=ocvaqGzvE2I

Task 3a:  How does an electrician test the ohmmeter to be sure it is working?

Answer:

_____________________________________________________________

Task 3b:  How does an electrician ‘zero’ out the analog ohmmeter?

Answer:

_____________________________________________________________

Task 3c: Using a meter reading at the two terminals, how do you know if a stove element is working?

Answer:

_____________________________________________________________

Task 3d: Draw a line to the dial to be used when ‘zeroing’ this analog meter.

Answer: No written response required here.

Task completed:  Yes:

Answers

Task 1a:  What colour is used to measure the ohms of a circuit?

Answer: Green

Task 1b:  Circle on the picture where the meter setting dial should be set to measure a 9 Volt battery.

Answer: 9 V (green circle)

Task 1c:  Electricians always set the

meter slightly higher than the expected

voltage being measured.  Circle on the

picture where the dial should be set to

measure a 240 volt AC circuit.

Answer: 250 (blue circle)

Task 1d:  Batteries are considered in working order at 70% voltage or better.  A 9 Volt battery reading shows the needle at about 43%.  In what range will the needle point?

Answer: Depending on which sensor image the learner views, the needle will either point to “Replace” or “Bad” range. Either answer is acceptable.

_____________________________________________________________

Go to the Lockout/Tag out page on the Canadian Centre for Occupational Health and Safety website:   http://www.ccohs.ca/oshanswers/hsprograms/lockout.html

Task 2a:  List three key pieces of information to be included on a tag?

Answer:

- Why the lockout/tag out is required (repair, maintenance, etc.).

- Time of application of the lock/tag.

- The name of the authorized person who attached the tag and lock to the system.

Task 2b:  An electrician uses a voltmeter to ensure the power to a circuit is safe to complete the lock out/tag out procedure.  Explain what the voltage reading should be and why this is the case.

Answers may vary but should be similar to: The reading should show 0 volts.  This has to be the reading because there should not be any electrical power at the lock out/tag out site.  This is for safety reasons.

_____________________________________________________________

Watch this video about using digital and analog ohmmeters:  http://www.youtube.com/watch?v=ocvaqGzvE2I

Task 3a:  How does an electrician test the ohmmeter to be sure it is working?

Answer: Turn the meter to the lowest setting. Place the two leads across the switch. The ohmmeter should go to 0.

Task 3b:  How does an electrician ‘zero’ out the analog ohmmeter?

Answer: Touch the two leads together and adjust the dial until the ohmmeter reaches 0 ohms. Then take the leads and place them across the thermostat. It should go to zero.

Task 3c: Using a meter reading at the two terminals, how do you know if a stove element is working?

Answer: The ohmmeter should display some value if the stove element is working.

Task 3d: Draw a line to the dial to be used when ‘zeroing’ this analog meter.

Answer:

# Performance Descriptors

| Levels | Performance Descriptors | Needs Work | Completes task with support from practitioner | Completes task independently |
| --- | --- | --- | --- | --- |
| A1.3 | integrates several pieces of information from texts |  |  |  |
|  | manages unfamiliar elements (e.g. vocabulary, context, topic) to complete tasks |  |  |  |
|  | skims to get the gist of longer texts |  |  |  |
|  | infers meaning which is not explicit in texts |  |  |  |
|  | uses organizational features, such as headings, to locate information |  |  |  |
|  | follows the main events of descriptive, narrative, informational and persuasive texts |  |  |  |
|  | identifies sources, evaluates and integrates information |  |  |  |
| A2.2 | uses layout to locate information |  |  |  |
|  | makes connections between parts of documents |  |  |  |
|  | makes low-level inferences |  |  |  |
| A3 | extract information from films, broadcasts and presentations |  |  |  |
| C3.1 | recognizes values in number and word format |  |  |  |
|  | understands numerical order |  |  |  |
| D.1 | follows simple prompts |  |  |  |
|  | follow apparent steps to complete tasks |  |  |  |
|  | interprets brief text and icons |  |  |  |
|  | locates specific functions and information |  |  |  |

This task: Was successfully completed        Needs to be tried again

Learner Comments:

Instructor (print):						Learner (print):

__________________                                         _________________